For the Realtek 8822ce driver on Linux, it's best to first try updating your system and kernel, as the driver is often included in newer versions. If that doesn't work, you will need to compile and install the driver manually from a source like GitHub. You will need an internet connection via a wired Ethernet cable or a USB tethered phone to download the necessary files and build the driver.

Step 1: Update your system and kernel

- Open your terminal and run your distribution's update command (e.g., sudo apt update && sudo apt upgrade for Debian/Ubuntu).

- Install the latest available kernel for your distribution. On some systems, you can do this through a "Linux Kernels" tool in the update manager or using the command line, as shown in this Linux Mint forum post.

Step 2: Compile and install the driver from source

- Install build tools and dependencies: Use your package manager to install necessary tools. For example, on Debian/Ubuntu, you can use:
sudo apt install build-essential git dkms.

- Download the driver: Clone a repository from GitHub. One option is:
git clone https://github.com/juanro49/rtl88x2ce-dkms.git.
Another popular option is https://github.com/tomaspinho/rtl8821ce, which supports the 8822ce chip as well.

- Install using DKMS: Follow the instructions in the repository's README file. The general process is to copy the files and then use dkms to build and install the module. A simplified example from the rtl88x2ce-dkms repository is:

bash

cd rtl88x2ce-dkms

sudo mkdir /usr/src/rtl88x2ce-35403

sudo cp -Rv * /usr/src/rtl88x2ce-35403/

sudo dkms add -m rtl88x2ce -v 35403

sudo dkms build -m rtl88x2ce -v 35403

sudo dkms install -m rtl88x2ce -v 35403

(Note: The version number 35403 may need to be changed based on the repository version).

- Load the new module: After installation, load the driver:
sudo modprobe rtl88x2ce.

- Reboot: Restart your computer to ensure the new driver is loaded correctly.

If you need to do this again after a kernel update

- Keep a copy of the driver files and the installation instructions on a USB drive for easy reinstallation.

- After a kernel update, repeat the "Install using DKMS" and "Load the new module" steps.